SMARTMINE

Intelligent Mine-Safety Monitoring Rover

Professional Project Description • Rover-Only Prototype

Figure 1. Overall SmartMine system architecture.

# 1. Executive Summary

SmartMine is a compact, remotely monitored robotic rover designed to support safer inspection of hazardous or difficult-to-access mining environments. The rover combines obstacle detection, environmental sensing, audible and visual alerts, wireless communication, and live camera monitoring in a single mobile platform. The current prototype is intentionally rover-only: four BO geared DC motors drive the wheels, while an Arduino Uno performs local control and an ESP32/ESP32-CAM provide wireless communication and visual monitoring.

The central engineering objective is to keep a person away from potentially unsafe areas during an initial inspection. The rover can approach an area, observe it through the ESP32-CAM, detect nearby obstacles, display status locally, and transmit useful information to a laptop dashboard.

# 2. Problem Statement

Mining environments can contain uneven ground, confined spaces, obstacles, poor visibility, and potentially hazardous atmospheric conditions. Sending a person into an unfamiliar area for an initial inspection can increase exposure to these hazards. SmartMine addresses this problem by providing a small mobile inspection platform capable of collecting basic sensor and video information remotely.

# 3. Project Objectives

- Build a stable four-wheel rover using the existing chassis and BO geared motors.

- Detect nearby obstacles using an HC-SR04 ultrasonic sensor and two IR sensors.

- Provide environmental/hazard sensing through a suitable gas or air-quality sensor.

- Provide immediate local warnings using an OLED display and buzzer.

- Add ESP32-CAM live video for visual inspection.

- Use ESP32 Wi-Fi communication to send rover information to a laptop dashboard.

- Create a modular platform that can later support additional sensors and analytics.

# 4. Hardware Architecture

| Component | Role | Interface / Connection | Purpose |
| --- | --- | --- | --- |
| Arduino Uno | Main local controller | Digital/analog I/O | Reads sensors and controls rover logic |
| ESP32 | Wireless controller | Wi-Fi / serial as designed | Communication with laptop/dashboard |
| ESP32-CAM | Camera subsystem | Wi-Fi | Live visual monitoring |
| HC-SR04 | Distance sensor | Trigger/Echo | Front obstacle detection |
| IR sensors ×2 | Proximity detection | Digital | Left/right obstacle awareness |
| Gas / hazard sensor | Environmental sensing | Analog/digital | Detects selected atmospheric parameter |
| OLED 128×64 | Local display | I²C | Status, sensor values and warnings |
| Buzzer | Local alarm | Digital output | Audible warning |
| Motor driver | Power interface | Arduino control inputs | Drives four BO geared motors |
| BO geared DC motors ×4 | Drive system | Motor driver outputs | Moves the rover |

# 5. Physical Layout

Figure 2. Suggested top-view placement on the existing four-wheel chassis.

The ESP32-CAM should be positioned at the front/top of the chassis with a clear forward field of view. The ultrasonic sensor should face forward and remain unobstructed. The IR sensors can be placed toward the front-left and front-right areas. The controller and power electronics should remain near the center of the chassis to keep wiring short and reduce the chance of mechanical interference.

# 6. Operating Principle

Power-up: The rover initializes the Arduino, ESP32, ESP32-CAM, display and sensor interfaces.

Sensing: The ultrasonic, IR and environmental sensors continuously collect measurements.

Decision: The Arduino evaluates sensor states and applies programmed movement/alert logic.

Movement: The motor driver supplies the required drive signals to the four BO geared motors.

Local warning: The OLED displays key readings/status and the buzzer provides an audible warning when programmed thresholds are reached.

Visual monitoring: The ESP32-CAM captures the forward scene and provides a Wi-Fi video stream.

Remote monitoring: The ESP32 communicates with the laptop dashboard for telemetry/control functions.

# 7. Data and Communication Flow

Figure 3. High-level control, telemetry and video flow.

# 8. ESP32-CAM Integration

The ESP32-CAM is added as the visual subsystem rather than replacing the Arduino. Its primary role is to provide a live view of the rover's surroundings. This is particularly useful because a distance sensor can indicate that an obstacle exists but cannot provide the same visual context as a camera.

- Mount the camera above the front section of the chassis.

- Keep the lens clear of the ultrasonic sensor and chassis structure.

- Use the ESP32-CAM's Wi-Fi capability for the video stream.

- Present the video beside sensor telemetry on the laptop dashboard where practical.

- For future development, captured frames can be processed for visual obstacle or hazard classification.

# 9. Software Architecture

The software should be organized into separate modules so that sensing, motion control, communication, and display functions can be tested independently.

- Sensor module: reads HC-SR04, IR sensors and the selected gas/environmental sensor.

- Motion module: converts navigation decisions into left/right motor commands.

- Safety module: checks obstacle and hazard conditions and triggers alerts.

- Display module: updates the OLED with sensor values and system state.

- ESP32 communication module: handles wireless commands and telemetry.

- ESP32-CAM module: provides the camera stream.

- Laptop dashboard: displays video, telemetry and alert status; it can later store readings for analysis.

# 10. Key Features

| Feature | Engineering benefit |
| --- | --- |
| Four-wheel mobility | Uses the existing geared BO motors and wheels for simple, robust ground movement. |
| Obstacle awareness | Combines ultrasonic and IR sensing for complementary proximity detection. |
| Visual inspection | ESP32-CAM adds live video to the sensor-based view of the environment. |
| Local alerts | OLED and buzzer provide immediate status and warnings on the rover. |
| Wireless monitoring | ESP32 provides a path to remote telemetry and control. |
| Modular expansion | The chassis can accept additional sensors, lighting, storage or processing modules. |

# 11. Prototype Testing Plan

- Test each BO motor independently before connecting the complete drivetrain.

- Verify wheel direction and left/right control at low speed.

- Check HC-SR04 distance readings against known distances.

- Test each IR sensor independently and verify its active state.

- Test the OLED and buzzer using simulated sensor conditions.

- Verify ESP32 Wi-Fi communication before integrating dashboard functions.

- Test the ESP32-CAM stream separately and then combine it with telemetry.

- Run integrated tests on a clear, controlled surface before moving to more complex terrain.

- For any future field demonstration, use appropriate adult/teacher supervision and follow the site's safety rules.

# 12. Expected Output

The completed prototype should demonstrate a four-wheel SmartMine rover that can move under remote/local control, detect nearby obstacles, monitor a selected environmental parameter, issue local warnings, and provide live camera observation through the ESP32-CAM. A laptop dashboard can combine the video feed with sensor values and system status.

# 13. Future Development

- Add a higher-quality camera or pan/tilt camera mount.

- Introduce data logging for sensor measurements and event timestamps.

- Add computer-vision algorithms for visual obstacle recognition.

- Add map/route visualization for larger test environments.

- Add additional environmental sensors appropriate to the intended mining demonstration.

- Develop a more robust communication link for longer-range controlled testing.

- Add autonomous navigation after reliable manual and sensor-level operation has been demonstrated.

# 14. Conclusion

SmartMine is a practical robotics platform that combines mobility, sensing, visual monitoring and wireless communication into one rover. The rover-only configuration keeps the prototype mechanically straightforward while still demonstrating the project's core mining-safety concept. The addition of the ESP32-CAM strengthens the system by giving the operator direct visual information alongside sensor data, creating a foundation for future remote inspection and computer-vision capabilities.

SMARTMINE • ONE ROVER • MULTIPLE SENSING MODALITIES • REMOTE VISUAL MONITORING